Table of Contents

1 Introduction	3

1.1 Historical Perspective	3

1.2 Motivation	3

1.3 Conceptual Overview if Experiment	4

2 Hypothesis, Objective & success Criteria	5

2.1 Hypothesis	5

2.2 Objective	5

2.3 Success Criteria	5

3 Literature Review	6

3.1 Recent Relevant Research	6

3.2 Solar Aircraft Design	6

3.3 Vehicle Performance Predictions	8

3.4 Propulsion System Design	9

4 Technical Approach	10

4.1 Experiment Description	10

4.2 Apparatus Description	10

4.3 Measurement Description	12

5 Appendix	14

5.1 Appendix A	14

6 List of Resources	16

Index of Figures

Figure 1: Proposed Aircraft Configuration	11

Figure 2: NG01 Airfoil with Proposed Array Integration	12

Figure 3: I x V characteristic curves of a photovoltaic cell	14

Figure 4: P x V characteristics of photovoltaic cell for different irradiance levels	14

Figure 5: Experimental results with and without MPPT	15

Index of Tables

Table 1: Test Matrix	13

- 1 Introduction

- 1.1 Historical Perspective

The birth of solar aviation came in the mid-1970s when rising oil prices and growing environmental concerns spurred the need for sources of clean alternative energy.  The first solar powered aircraft, the unmanned Sunrise I, flew in the winter of 1974-75. The aircraft weighed 27.5 lbs, had a wingspan of 32 ft, and was powered by a 450 watt solar array. Its successor, Sunrise II, flew in the fall of 1975 using an upgraded 600 watt array and a lightened airframe. The array from Sunrise II was then used to power the world’s first man-carrying solar aircraft, Gossamer Penguin, which made many successful flights in the summer of 1980. The AeroVironment team responsible for Gossamer Penguin constructed the more robust Solar Challenger, which flew across the English Channel in 1981 powered by a 4000 watt array. This trend of larger vehicles with more powerful arrays continued through the next two decades. Most recently, AeroVironment has been working with its Pathfinder/Centurion/Helios series of unmanned aircraft. Helios, the largest aircraft in the series with a wingspan of 247 ft and a 42 kilowatt array, first flew in 2001 and has since set many altitude and duration records.1

- 1.2 Motivation

With the continually growing role of unmanned aerial vehicles (UAV’s) in civil and military operations, new longer-duration UAV applications are being conceived. Solar aircraft provide a promising solution to the duration demands placed on UAV’s in missions where current battery technology proves insufficient. As previously outlined, solar aircraft have been in existence for many years, yet most of the existing vehicles are powered by multimillion dollar arrays.* This high array cost eliminates the vehicle’s disposability, making large solar powered UAV’s unsuitable for high-risk missions. Furthermore, high array costs make current solar UAV’s too expensive for most privately-funded operations to own or operate. In light of these factors, the need for smaller, low-cost solar vehicles is evident. While some hobbyists and universities have explored this domain of low-cost solar flight, most research has centered on achieving flight rather than developing a high performance aircraft. This experiment intends to develop a low cost solar powered UAV and thoroughly characterize its performance, with the intent to demonstrate that high performance can still be obtained on a limited budget.

- 1.3 Conceptual Overview if Experiment

The primary performance advantage of solar powered UAV’s over their gasoline or battery powered counterparts is increased flight duration. This flight duration is determined by the minimum solar inclination angle (SIA) and corresponding time of day at which the aircraft can sustain flight. If the aircraft can operate at low SIA’s, when the solar array is outputting a fraction of its maximum midday power, it can take off earlier in the morning and land later in the evening. The proposed experiment will evaluate the SAI operating range of a low-cost solar-powered UAV. Flight tests to assess vehicle and solar array performance predictions will be carried out using only solar power.  Thus, the minimum SIA will be defined as the sun angle at which the solar array can no longer provide sufficient power to sustain level flight.*

- 2 Hypothesis, Objective & success Criteria

- 2.1 Hypothesis

A $500 solar array can be integrated into a small-scale UAV and provide sufficient power to sustain flight at a solar inclination greater than ___o. [TBD after analysis]

- 2.2 Objective

Modify the airframe of an appropriate commercially available model aircraft to accept a solar propulsion system. Flight-test the instrumented aircraft to attain the necessary data to assess the hypothesis.

- 2.3 Success Criteria

Assess whether the vehicle is able to sustain flight at or below the predicted minimum solar inclination angle of ___o determined during aircraft performance analysis.

- 3 Literature Review

- 3.1 Recent Relevant Research

While large UAV’s like the Helios are at the forefront of solar aircraft design, the development of successful low-cost solar vehicles in recent years has been limited. These low-cost vehicles have met with minimal success because, to meet the low cost requirement, cheaper solar cells are used to construct the solar array. These cheaper cells have greatly reduced efficiency and higher mass compared to space-grade cells that power vehicles like Helios. This further reduces the available power for a vehicle that is already operating with a remarkably low power margin. Despite, these challenges two private researcher/hobbyists have designed relatively high performance solar powered UAV’s at the scale of interest for this proposed experiment. David Beck designed and built Solar Solitude using commercially available solar cells and model aircraft electronics. The aircraft weighed 4.4 lbs, had a 106 in. wingspan, and was powered by a 63 watt array. This puts the aircraft in the size range that this experiment intends to explore. David first flew the aircraft in 1998, and went on to achieve flight times exceeding 6 hours.2 Another hobbyist, Wolfgang Schaeper of Germany, used a modified model sailplane to achieve a solar powered flight time of 11 hours and 34 minutes in 1997 (the current duration record for a solar powered model airplane).3

- 3.2 Solar Aircraft Design

Due to the unique power limitations of solar aircraft, special design considerations must be made to produce a viable vehicle. “Sun Powered Aircraft Design” by MacCready, Lissaman, Morgan, and Burke provides valuable insight into this unique design process. The MacCready paper chronicles the design and testing of the two first man-carrying solar powered vehicles, Gossamer Penguin and Solar Challenger. While these vehicles are far outside of the scope of this experiment, the information presented regarding special design considerations for solar powered aircraft will be particularly useful during the design of test vehicle. In traditional minimum energy vehicle design, high-aspect-ratio wings are desirable for their low induced drag penalty. However, on a solar aircraft, increased wing area corresponds to increased available power since the wing is covered by solar cells. Thus, to generate more power and keep the wingspan at a reasonable length the traditional high-aspect-ratio constraint is relaxed.4 To account for this unique relationship between wing area and available power, the primary design variables used to optimize the wing geometry of the test vehicle will be aspect-ratio, wing area, and percentage of wing area covered by solar cells.

Since the solar cells are fragile the aft 85% of Solar challenger’s airfoil was designed to be a flat surface. This allowed for easy and safe installation of the solar cells. Making a change like this to the airfoil geometry has a potentially devastating effect on its aerodynamic properties; thus detailed airfoil analysis is necessary to facilitate successful cell integration.4 Similar preliminary tests have been performed on the NG01 airfoil proposed for this experiment. Faceting the aft portion of the airfoil’s surface will allow for easy cell integration (Figure 2,) and analysis of the drag polars in XFOIL have shown that the faceting produces no adverse effects. Other points of interest outlined in the paper include the need for thermal expansion joints between cells to prevent fracture, the need for adequate power margin for maneuvering in case of emergency, and the need to wire cell packs in parallel to avoid overloading adjacent cells in the event of a single cell failure.4

- 3.3 Vehicle Performance Predictions

To meet the success criteria, the test vehicle must fly at or below a minimum predicted SIA. To establish this minimum SIA, detailed vehicle performance predictions need to be performed. “The Sunriser-A Design study in Solar Powered flight” by Patrick Berry details the prediction and validation of these aircraft performance parameters.

The Sunriser was an on-paper design exercise to create an optimized man-carrying solar vehicle. To asses the design’s viability, Patrick Berry’s team at Linkoping University, created the “Solaris” computer program, which uses a series of simple weight, power, and aerodynamic models to determine if a solar powered design will be flyable. To test the “Solaris” program, the team created a small UAV, the Sunrazor, then test flew it to determine if the actual flight performance matched the performance predicted by “Solaris.” The Sunrazor flew with marginal success, confirming the predictions made by “Solaris.”5

This paper is noteworthy because, it shows the ability to predict the performance of solar powered aircraft with a reasonable degree of confidence. It also provides a host of simple equations that can be used to create an accurate performance model for the test vehicle. Also the Sunrazor’s marginal controllability and low power margin described in the text serve as a cautionary note.5 The test aircraft must not be designed solely for minimum power requirements. A large power margin and a sound, controllable aircraft platform are vital for success.

- 3.4 Propulsion System Design

Propulsion system design is of great importance in this experiment due to the inherently low efficiency of solar propulsion. As described in Santos and Antunes’ paper, “Maximum Power Point tracker for PV Systems,” this already low efficiency can be further reduced by temperature increases or by drawing too much current from the solar cells.6  “A PV system is a non-linear power source, i.e. its output current/power depends on the terminal operating voltage. On the other hand, the maximum power generated by the system changes with solar radiation and temperature.”6 This relation ship between the maximum cell output power point and current draw can be seen in Appendix A. Normally a battery buffer is used to regulate the excess current created by the cells and prevent over drawing. However, since it is the intent of this experiment to operate without a battery buffer, another solution must be found. Thus, to keep the cells operating at their maximum power point (MPP,) a maximum power point tracking (MPPT) device must be used. The paper describes the design and implementation of a single chip 80C51 MPPT. The MPPT constantly monitors cell current and voltage draws, then generates a PWM signal that is relayed through a conventional eclectic motor speed controller to adjust the motor draw accordingly.6 A MPPT such as this could be fabricated or purchased for use in this experiment. The MPPT would greatly increase the solar array’s efficiency, allowing the aircraft to sustain flight at lower solar inclination angles.

- 4 Technical Approach

- 4.1 Experiment Description

Upon completion of the airframe, flight tests will occur in large open area, free of objects that could obstruct the flight path or occlude the setting sun. Davis Field in Sudbury, MA is the current candidate test site. The test will start approximately 45-60 minutes before the sun reaches the minimum SIA specified in the hypothesis. This will guarantee that the solar array will be producing adequate power to sustain flight at the outset of the test. The aircraft will then be flown around a predefined circuit until the solar array can no longer provide enough power to sustain level flight (this will necessitate a forced landing). An onboard EagleTree™ flight data recorder will monitor the elapsed flight time and the aircraft’s sink rate. This data can then be used to determine the time (and corresponding SIA) during the test at which the aircraft could no longer sustain level flight, i.e. positive sink rate. This will provide sufficient information to assess whether the vehicle is able to sustain flight at or below the predicted minimum SIA determined during the preliminary analysis. The test will be repeated on multiple days to account for inevitable variations in weather conditions, particularly cloud cover and radiation index.

- 4.2 Apparatus Description

A commercially available model aircraft will be modified to serve as the test apparatus. These modifications will fall under two primary categories, solar propulsion system integration and wing sizing. Due to the unique constraints of solar aircraft design, parameters such as wing area, aspect ratio, and solar array area are best determined through a multi-disciplinary optimization (MDO). These parameters are varied to produce a wing configuration that maximizes power margin, power available/ power required. Power margin was chosen as the target design variable as it directly correlates to the minimum SIA at which the aircraft can operate. There is a geometric efficiency that is a function of SIA. This geometric efficiency reduces the available power as the SIA decreases. Thus, a higher power margin will allow the aircraft to fly at lower minimum SIA.  The possible aircraft configuration shown in Figure 1 below is a product of preliminary wing MDO calculations. A NG01 airfoil has been selected for the test aircraft due to its high lift-to-drag ratio over a high speed range. Modifications to the airfoil to facilitate incorporation the solar array can be seen in Figure 2.

- Figure 1: Proposed Aircraft Configuration

- Figure 2: NG01 Airfoil with Proposed Array Integration

- 4.3 Measurement Description

During each test flight the onboard EagleTree™ flight data recorder will record the elapsed test time, the motor power draw, and the aircraft’s sink rate. At the beginning of each test, the time of day and the latitude and longitude coordinates of the test site will be recorded. The elapsed test time from the data recorder can be added to the test start time to determine the exact time of day of each test data point. This time of day can be entered into an online solar position calculator* along with the current date and lat/long coordinates to determine the exact SIA for each test data point. These SIA’s are the independent variable in the test matrix test matrix below (Table 1.), and each corresponds to an experimentally measured sink rate.

| SIA (deg) | Θ1 | Θ2 | Θ3 | Θ4 | Θ5 | …. | Θn |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sink Rate (ft/s) | W1 | W2 | W3 | W4 | W5 | .... | Wn |

- Table 1: Test Matrix

When the sink rate takes on a consistent positive value, the aircraft is no longer sustaining level powered flight, and thus the system is operating at the minimum experimental SIA.  Motor power data can be used to verify this loss of sustained flight; if sink rate assumes a positive value but motor power goes unchanged, it can be assumed that the airplane is in a downdraft or powered dive. However, if motor power drops when sink rate assumes a positive value it is clear that the array is losing power as well, corresponding to a loss in solar energy as the sun approaches the minimum SIA. To verify or refute the hypothesis, these results can be compared to the minimum SIA that will be predicted during the analysis of the final aircraft configuration.

One parameter of particular note will be the radiation index for each test day. Variations in this parameter over multiple test days will have to be accounted for during the data analysis. It will determine the maximum power output of the array for a given test day, altering the aircraft’s power margin. Thus, it will need to be treated like a tare correction that slightly shifts the measured minimum SAI.

- 5 Appendix

- 5.1 Appendix A

- Figure 3: I x V characteristic curves of a photovoltaic cell6

- Figure 4: P x V characteristics of photovoltaic cell for different irradiance levels6

- Figure 5: Experimental results with and without MPPT6

- 6 List of Resources

1. Roberts, C., Vaughan, M., Bowman, W. J., “Development of a Solar Powered Micro Air Vehicle,” 40th Aerospace Sciences Meeting and Exhibit, 14-17 January 2002, Reno, Nevada, AIAA 2002-0703.

2. Solar Solitude Web site: http://personalpages.tds.net/~dbeck/ Date accessed, March 28, 2006.

3. FAI Web site: http://records.fai.org/models/current.asp?id=199/ Date accessed, March 28, 2006.

4. MacCready, P. B., Lissaman, P.B.S, Morgan, W. R., and Burke, J.D., “Sun Powered Aircraft Design,” AIAA 1981 Annual Metting and Technical Display, Long Beach, CA, May 12-14, 1981, AIAA-81-0916.

5. Berry, Patrick, “The Sunriser-A Design Study in Solar Powered Flight,” AIAA 2000 World Aviation Conference, San Diego, CA, Oct. 10-12, 2000, A00-45009.

6. Santos, Jancarle L., and Antunes, Fernando L.M., “Maximum Power Point Tracker for PV Systems,” World Climate and energy Event, Rio de Janeiro, Brazil, Dec. 1-5, 2003.

* Taken directly from Version I project description.

* Taken directly from Version I project description.

* http://solardat.uoregon.edu/SolarPositionCalculator.html